LAB1: DAD

With 5 inputs

library ieee;

use ieee.std_logic_1164.all;

-- define entity

entity myLogicFunction_tb is

end entity;

architecture ben of myLogicFunction_tb is

-- define component

component myLogicFunction is port(

a,b,c,d,e     : in  std_logic;

o         : out std_logic);

end component;

-- These are the internal wires

signal a,b,c,e,d,o : std_logic;

constant N: natural:=5;

begin

-- uut is the instance name

uut : myLogicFunction port map(a => a, b => b, c => c,d =>d, e=>e, o => o);

stim : process

begin

a <= '0'; b <= '0'; c <= '0'; d<='0'; e<='1';

wait for 10 ns;

assert ((o = '0'))

report "test failed for input combination 000" severity error;

a <= '1'; b <= '1'; c <= '1'; d<='1'; e<='0';

wait for 10 ns;

assert ((o = '1'))

report "test failed for input combination 111" severity error;

a <= '0'; b <= '1'; c <= '0'; d<='1'; e<='1';

wait for 10 ns;

assert ((o = '0'))

report "test failed for input combination 010" severity error;

end process;

end ben;

library ieee;

use ieee.std_logic_1164.all;

entity myLogicFunction is port (

a : in std_logic;

b : in std_logic;

c : in std_logic;

d :in std_logic;

e :in std_logic;

o : out std_logic

); end myLogicFunction;

architecture arch of myLogicFunction is

begin

o <= (a and b and c) and (d or e);

end arch;

task2: 5 inputs
library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

-- define entity

entity myLogicFunction_tb is

end entity;

architecture ben of myLogicFunction_tb is

-- define component

component myLogicFunction is port(

a,b,c,d,e     : in  std_logic;

o         : out std_logic);

end component;

-- These are the internal wires

signal a,b,c,e,d,o : std_logic;

constant N: natural:=5;

begin

-- uut is the instance name

uut : myLogicFunction port map(a => a, b => b, c => c,d =>d, e=>e, o => o);

stim : process

variable vec : unsigned(N-1 downto 0);

begin

for i in 0 to 2**N - 1 loop

vec := to_unsigned(i,N);

a <= vec(0);

b <= vec(1);

c <= vec(2);

d <= vec(3);

e <= vec(4);

wait for 10ns;

end loop;

wait;

end process;

end ben;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity TopLevel is port (

a: in std_logic;

b: in std_logic;

c: in std_logic;

o1: out std_logic;

o2: out std_logic;

o3: out std_logic

); end TopLevel;

architecture Bh3 of TopLevel is

signal t1, t2, t3: std_logic;

begin

m1: entity work.I1 port map(a=> a, b => b, c=>c, o1=>t1, o2=>t2, o3=>t3);

m2: entity work.I2 port map(a=>t1, b=>t2, c=>t3, o1=>o1, o2=>o2, o3=>o3);

end Bh3;

library ieee;

use ieee.std_logic_1164.all;

entity TopLevel_tb is

end entity;

architecture Behavioral of TopLevel_tb is

signal a, b, c : std_logic := '0';

signal o1, o2, o3 : std_logic;

begin

-- Direct entity instantiation (simplest & correct)

uut1 : entity work.TopLevel

port map (

a  => a,

b  => b,

c  => c,

o1 => o1,

o2 => o2,

o3 => o3

);

stim : process

begin

a <= '0'; b <= '0'; c <= '0';

wait for 10 ns;

a <= '0'; b <= '0'; c <= '1';

wait for 10 ns;

a <= '0'; b <= '0'; c <= '0';

wait for 10 ns;

a <= '0'; b <= '1'; c <= '1';

wait for 10 ns;

a <= '0'; b <= '0'; c <= '0';

wait for 10 ns;

a <= '0'; b <= '0'; c <= '1';

wait for 10 ns;

wait; -- stop simulation

end process;

end architecture Behavioral;